Product name: 0.9m 2.4GHz Parabolic Antenna

Product model: GL-DY2425V24

Applications:

2.4GHz, WLAN, WIFI Wireless System

Point-to-point, point-to-multipoint systems

Features:

High gain, high front-to-back ratio

Anti-aging Surface Coati

| Electrical Specifications |  |
| --- | --- |
| Frequency Range-MHz | 2400-2500 |
| Polarization | Vertical |
| VSWR | ≤1.5 |
| Gain-dBi | 24 |
| Horizontal beam width | 10° |
| Vertical beam width | 14° |
| Front-to-back ratio-dB | ≥30 |
| Input Impedance-Ω | 50 |
| Maximum input power-W | 100 |
| Lightning protection | DC Ground |
| Mechanical Specifications |  |
| Input connector type | N-K |
| Dimensions-m | 0.9×0.6 |
| Antenna weight-kg | 3 |
| Reflector material | Aluminum |
| Reflecting surface color | Gray |
| Mechanical tilt | 0～20° |
| Operating temperature | -40°C to + 65 °C |
| Rated Wind Velocity-m/s | 60 |
| Mounting hardware - mm | φ35～φ50 |