Product Name:800M/2.6G/3.5G 3-band base station panel antenna with upward coverage

product model:GL8035G10

Features：

- A larger size means more antenna elements can be housed internally, resulting in a wider and farther radiating area and better directivity.

- Compatible with multiple different frequency bands

- Connector type, frequency band, size, gain, and mounting method can be customized according to different application scenarios.

| Electrical Specifications |  |  |  |
| --- | --- | --- | --- |
| Frequency Range | 750-803 | 2620-2690 | 3300-3600 |
| Gain (dBi) | 10±1 | 10±1 | 10±1 |
| VSWR | ≤1.5 |  |  |
| Front-to-back ratio- dB | ≥23 |  |  |
| Polarization | vertical |  |  |
| Horizontal beam width(°) | 65±8/65±8/65±8 |  |  |
| Vertical beam width (°) | 32±5/32±5/32±5 |  |  |
| Maximum input power (W) | 100 |  |  |
| Impedance (Ω) | 50 |  |  |
| Lightning protection | DC Ground |  |  |
| Mechanical Index |  |  |  |
| Dimensions (mm) | 750*160*80mm |  |  |
| Input connector type | 3*N-Female |  |  |
| Radome material | UPVC |  |  |
| Antenna weight-KGS | 2.5 |  |  |
| Usage scenarios | outdoor |  |  |
| Operating temperature (℃) | -40 ~ +85 |  |  |
| Humidity range (%) | 5~95 |  |  |