# PRODUCT SPECIFICATION

MODULE NO.                 GX-M310

CUSTOMER

ISSUED DATE

CUSTOMER SIGNATURE BAR

| CUSTOMER | ADMISSION | APPROVAL |
| --- | --- | --- |

Signed by                    Approved by

GX-M310

## 1. Product Information

### 1.1 Product Name

GX-M310

### 1.2 Product Description

GX-M310  is  a  compact,  high  performance,  and  low   power  consumption GNSS engine board. It uses the chipset which can track up to 66 channels at a time and perform fast TTFF in weak signal environments.

### 1.3 Product Features

MediaTek high sensitivity solution    Support 99-channel GPS&BEIDOU

Ultra low power consumption Fast TTFF at low signal level

Built-in 12 multi-tone active interference canceller

Free  hybrid  ephemeris  prediction  to  achieve  faster  cold start Built-in data logger

Built-in DC/DC converter to save power Up to 10 Hz update rate

±11ns high accuracy time pulse (1PPS)

Capable of SBAS (WAAS, EGNOS, MSAS, GAGAN) Support Japan QZSS

Indoor and outdoor multi-path detection and compensation Small form factor 10.1 * 9.7 * 2.5mm ±0.2mm

SMD    type    with    stamp holes RoHS compliant

### 1.4 Product Application

Automotive navigation Personal positioning

Fleet management

1

GX-M310

Mobile  phone  navigation Marine navigation

## 2. Technical Specifications

|  | GPS Receiver |  |
| --- | --- | --- |
| Chip | MediaTek MT3333 (Flash Version) |  |
| Frequency | GPS, GALILEO, QZSS:L1 1575.42MHz, C/A code BEIDOU: B1 1561.098MHz |  |
| Channels | Support 99 channels |  |
| Update rate | 1Hz default, up to 10Hz |  |
| Sensitivity | Tracking | -162dBm, up to -165dBm (with external LNA) |
|  | Cold Start | -143.5dBm, up to -148dBm (with external LNA) |
| Acquisition Time | Hot start (Open Sky) | < 1s (typical) |
|  | Hot start (Indoor) | < 30s |
|  | Cold Start (Open Sky) | 32s (typical) without AGPS |
|  |  | < 15s (typical) with AGPS (hybrid ephemeris prediction) |
| Position Accuracy | Autonomous | 3m (2D RMS) |
|  | SBAS | 2.5m (depends on accuracy of correction data) |
| Max. Altitude | < 50,000 m |  |
| Max. Velocity | < 515 m/s |  |
| Protocol Support | NMEA 0183 ver 4.10 | 9600 bps, 8 data bits, no parity, 1 stop bits (default) 1Hz: GGA, GLL, GSA, GSV, RMC, VTG |
|  | Physical Characteristic |  |
| Type | 18 pin stamp holes |  |
| Dimensions | 10.1mm * 9.7 mm * 2.5mm ±0.2mm |  |

## 3. Technical Information

Module Pin Assignment

2

GX-M310

| Pin NO. | Pin Name | I/O | Remark |
| --- | --- | --- | --- |
| 1. | GND | G | Ground. |
| 2. | TXD | O | Data Output |
| 3. | RXD | I | Data Input |
| 4. | TIMEPULSE | O | 1 Pulse per second |
| 5. | NC | N | Not connected |
| 6. | V_BCKP | PWR | Backup battery supply voltage |
| 7. | NC | N | Not connected |
| 8. | VCC | PWR | Main power supply to the engine board. |
| 9. | VRESET | I | Reset |
| 10. | GND | G | Ground. |
| 11. | RF IN | RF | GPS antenna input |
| 12. | GND | G | Ground. |
| 13. | NC | N | Not connected |
| 14. | NC | N | Not connected |
| 15. | NC | N | Not connected |
| 16. | NC | N | Not connected |
| 17. | NC | N | Not connected |
| 18. | NC | N | Not connected |

Dimensions

3

GX-M310

## 4. NMEA 0183 Protocol

The NMEA protocol is an ASCII-based protocol, Records start with a $ and with carriage return/line feed. GPS specific messages all start with $GPxxx where xxx is a three-letter identifier of the message data that follows. BEIDOU specific messages all start with

$BDxxx where xxx is a three-letter identifier of the message data that follows. NMEA messages have a checksum, which allows detection of corrupted data transfers.

GX-M310 modules support the following NMEA-0183 messages: GGA, GLL,GSA, GSV, RMC and VTG.

### 4.1 NMEA output message

Table 3.1-1 NMEA output message

| NMEA | Description |
| --- | --- |
| GGA | Global positioning system fixed data |
| GLL | Geographic position - latitude/longitude |
| GSA | GNSS DOP and active satellites |
| GSV | GNSS satellites in view |
| RMC | Recommended minimum specific GNSS data |
| VTG | Course over ground and ground speed |

4

GX-M310

#### 4.1.1 GGA--- Global Positioning System Fixed Data

Table 3.1-2 contains the values for the following example:

$GPGGA,060406.000,2503.7148,N,12138.7451,E,2,17,0.71,116.7,M,15.3,M,0000,0000*6D

Table3.1- 2 GGA Data Format

| Name | Example | Units | Description |
| --- | --- | --- | --- |
| Message ID | $GPGGA |  | GGA protocol header |
| UTC Time | 060406.000 |  | hhmmss.sss |
| Latitude | 2503.7148 |  | ddmm.mmmm |
| N/S indicator | N |  | N=north or S=south |
| Longitude | 12138.7451 |  | dddmm.mmmm |
| E/W Indicator | E |  | E=east or W=west |
| Position Fix Indicator | 2 |  | See Table 5.1-3 |
| Satellites Used | 17 |  | Range 0 to 33 |
| HDOP | 0.71 |  | Horizontal Dilution of Precision |
| MSL Altitude | 116.7 | mters |  |
| Units | M | mters |  |
| Geoid Separation | 15.3 | mters |  |
| Units | M | mters |  |
| Age of Diff. Corr. | 0000 | second | Null fields when DGPS is not used |
| Diff. Ref. Station ID | 0000 |  |  |
| Checksum | *6D |  |  |
| <CR> <LF> |  |  | End of message termination |

Table 3.1-3 Position Fix Indicators

| Value | Description |
| --- | --- |
| 0 | Fix not available or invalid |
| 1 | GPS SPS Mode, fix valid |
| 2 | Differential GPS, SPS Mode, fix valid |
| 3-5 | Not supported |
| 6 | Dead Reckoning Mode, fix valid |

5

GX-M310

#### 4.1.2 GLL--- Geographic Position - Latitude/Longitude

Table 3.1-4 contains the values for the following example: $GNGLL,2503.7148,N,12138.7451,E,060406.000,A,D*46

Table 3.1-4 GLL Data Format

| Name | Example | Units | Description |
| --- | --- | --- | --- |
| Message ID | $GNGLL |  | GLL protocol header (GPGLL or GNGLL; GP indicates the device receives GPS satellites signal only and GN indicates the position is calculated with BEIDOU satellite signal) |
| Latitude | 2503.7148 |  | ddmm.mmmm |
| N/S indicator | N |  | N=north or S=south |
| Longitude | 12138.7451 |  | dddmm.mmmm |
| E/W indicator | E |  | E=east or W=west |
| UTC Time | 060406.000 |  | hhmmss.sss |
| Status | A |  | A=data valid or V=data not valid |
| Mode | D |  | A=autonomous, D=DGPS, E=DR, N=Data not valid, R=Coarse Position, S=Simulator |
| Checksum | *46 |  |  |
| <CR> <LF> |  |  | End of message termination |

#### 4.1.3 GSA---GNSS DOP and Active Satellites

Table 3.1-5 contains the values for the following example:

$GNGSA,A,3,22,21,18,12,24,25,14,15,193,,,,1.18,0.71,0.95*2C

$GNGSA,A,3,205,207,210,202,201,203,209,208,,,,,1.18,0.71,0.95*1C

Table 5.1-5 GSA Data Format

| Name | Example | Units | Description |
| --- | --- | --- | --- |
| Message ID | $GNGSA |  | GSA protocol header (GNGSA or GPGSA; GP indicates the device receives GPS satellites signal only and GN indicates the position is calculated with BEIDOU satellite signal).First row of GSA message contains GPS & QZSS satellites and second row of GSA message contains BEIDOU satellites. |
| Mode 1 | A |  | See Table 5.1-6 |

6

GX-M310

| Mode 2 | 3 |  | See Table 5.1-7 |
| --- | --- | --- | --- |
| ID of satellite used | 22 |  | Sv on Channel 1 |
| ID of satellite used | 21 |  | Sv on Channel 2 |
| N. |  |  | N. |
| ID of satellite used |  |  | Sv on Channel 12 |
| PDOP | 1.18 |  | Position Dilution of Precision |
| HDOP | 0.71 |  | Horizontal Dilution of Precision |
| VDOP | 0.95 |  | Vertical Dilution of Precision |
| Checksum | *2C |  |  |
| <CR> <LF> |  |  | End of message termination |

Table 3.1-6 Mode 1

| Value | Description |
| --- | --- |
| M | Manual- forced to operate in 2D or 3D mode |
| A | Automatic-allowed to automatically switch 2D/3D |

Table 3.1-7 Mode 2

| Value | Description |
| --- | --- |
| 1 | Fix not available |
| 2 | 2D |
| 3 | 3D |

#### 4.1.4 GSV---GNSS Satellites in View

Table 3.1-8 contains the values for the following example:

$GPGSV,4,1,13,19,81,216,46,03,71,010,45,27,63,031,45,23,52,245,44*7B $GPGSV,4,2,13,06,50,034,47,50,46,122,48,13,45,290,44,16,34,038,42*75 $GPGSV,4,3,13,11,28,184,35,07,15,320,35,07,15,320,35,31,10,124,39*73 $GPGSV,4,4,13,08,,,15*77

$BDGSV,2,1,08,08,79,307,38,07,73,100,40,10,70,345,37,03,65,190,39*6E $BDGSV,2,2,08,02,44,241,36,04,33,110,42,09,04,199,29,01,,,34*53

Table 3.1-8 GSV Data Format

| Name | Example | Units | Description |
| --- | --- | --- | --- |
| Message ID | $GPGSV |  | GSV protocol header(GPGSVL or BDGSV; GP indicates the device receives GPS satellites signal only and BD indicates the position is |

7

GX-M310

|  |  |  | calculated with BEIDOU satellite signal) |
| --- | --- | --- | --- |
| Total number of messages1 | 4 |  | Range 1 to 6 |
| Message number1 | 1 |  | Range 1 to 6 |
| Satellites in view | 13 |  |  |
| Satellite ID | 19 |  | Channel 1 (Range 01 to 237), GPS Satellites ID : 01~32,SBAS Satellites ID : 33~64, QZSS Satellites ID:193~196, &BEIDOU Satellites ID : 01~32 |
| Elevation | 81 | degrees | Channel 1 (Range 00 to 90) |
| Azimuth | 216 | degrees | Channel 1 (Range 000 to 359) |
| SNR (C/No) | 46 | dB-Hz | Channel 1 (Range 00 to 99, null when not tracking) |
| N. | N. | N. | N. |
| Satellite ID | 23 |  | Channel 4 (Range 01 to 237) , GPS Satellites ID : 01~32,SBAS Satellites ID : 33~64, QZSS Satellites ID:193~196, &BEIDOU Satellites ID : 01~32 |
| Elevation | 52 | degrees | Channel 4 (Range 00 to 90) |
| Azimuth | 245 | degrees | Channel 4 (Range 000 to 359) |
| SNR (C/No) | 44 | dB-Hz | Channel 4 (Range 00 to 99, null when not tracking) |
| Checksum | *7B |  |  |
| <CR> <LF> |  |  | End of message termination |

1.   Depending  on  the  number of satellites tracked multiple messages of GSV data may be required.

#### 4.1.5 RMC---Recommended Minimum Specific GNSS Data

Table 3.1-9 contains the values for the following example:

$GNRMC,060406.000,A,2503.7148,N,12138.7451,E,0.01,0.00,180313,,,D*78

Table 3.1-9 RMC Data Format

| Name | Example | Units | Description |
| --- | --- | --- | --- |
| Message ID | $GNRMC |  | RMC protocol header (GNRMC or GPRMC; GP indicates the device receives GPS satellites signal only and GN indicates the position is calculated with BEIDOU satellite signal) |
| UTC Time | 060406.000 |  | hhmmss.sss |

8

GX-M310

| Status | A |  | A=data valid or V=data not valid |
| --- | --- | --- | --- |
| Latitude | 2503.7148 |  | ddmm.mmmm |
| N/S Indicator | N |  | N=north or S=south |
| Longitude | 12138.7451 |  | dddmm.mmmm |
| E/W Indicator | E |  | E=east or W=west |
| Speed over ground | 0.01 | knots | True |
| Course over ground | 0.00 | degrees |  |
| Date | 180313 |  | ddmmyy |
| Magnetic variation |  | degrees |  |
| Variation sense |  |  | E=east or W=west (Not shown) |
| Mode | D |  | A=autonomous, D=DGPS, E=DR, N=Data not valid,R=Coarse Position, S=Simulator |
| Checksum | *78 |  |  |
| <CR> <LF> |  |  | End of message termination |

#### 4.1.6 VTG---Course Over Ground and Ground Speed

Table 3.1-10 contains the values for the following example: $GPVTG,0.00,T,,M,0.01,N,0.02,K,D*3B

Table 3.1-10 VTG Data Format

| Name | Example | Units | Description |
| --- | --- | --- | --- |
| Message ID | $GPVTG |  | VTG protocol header |
| Course over ground | 0.00 | degrees | Measured heading |
| Reference | T |  | True |
| Course over ground |  | degrees | Measured heading |
| Reference | M |  | Magnetic |
| Speed over ground | 0.01 | knots | Measured speed |
| Units | N |  | Knots |
| Speed over ground | 0.02 | km/hr | Measured speed |
| Units | K |  | Kilometer per hour |

9

GX-M310

| Mode | D |  | A=autonomous, D=DGPS, E=DR, N=Data not valid,R=Coarse Position, S=Simulator |
| --- | --- | --- | --- |
| Checksum | *3B |  |  |
| <CR> <LF> |  |  | End of message termination |

### 4.2 Operating Modes

Send commands to select the different mode of operation.

| Command | Operating Mode |
| --- | --- |
| PMTK353,0,0,0,0,1 | BD only mode |
| PMTK353,1,0,0,0,0 | GPS only mode |
| PMTK353,1,0,0,0,1 | GPS + BD mode |

### 4.3 Proprietary NMEA input/output message

Please refer to MTK proprietary message.

### 4.4 Examples to configure the update rate of GNSS module

The  GNSS  module  supports  up  to  10Hz  update  rate  that  user  can  configure  by  issuing software commands. Note that  the configurations by software commands are stored  in the battery-backed  SRAM  that  is  powered  through  VBACKUP   pin.   Once  it  drains  out,  the default/factory settings will be applied.

Due to the transmitting capacity per second of the current baud rate, GNSS module has to be changed to higher baud rate for high update rate of position fix. The user can use the following software commands to change baud rate.

| Baud rate | Software command |
| --- | --- |
| default | $PMTK251,0*28<CR><LF> |
| 4800 | $PMTK251,4800*14<CR><LF> |
| 9600 | $PMTK251,9600*17<CR><LF> |
| 19200 | $PMTK251,19200*22<CR><LF> |
| 38400 | $PMTK251,38400*27<CR><LF> |
| 57600 | $PMTK251,57600*2C<CR><LF> |
| 115200 | $PMTK251,115200*1F<CR><LF> |

10