Model: HT-CPE500 OPENWRT 300Mbps Wireless N 4G LTE Router

Features

● Cutting-edge 4G network - share internet access with up to 30 Wi-Fi devices and enjoy download speeds of up to 150 Mbps and Upload speed to 50Mbps ● Plug a SIM card and play - no configurations needed, compatibility of SIM cards in 100+ countries are assured by years of field tests ● Two advanced LTE antennas and Two WIFI antenna - enjoy stable and efficient connections to every device thanks to the two external antennas ● Wi-Fi router mode - plug an Ethernet cable into the LAN/WAN port for flexible access as a backup option if you can't get a 4G connection Description 1) Support 3G, 4G LTE wireless network technology. 2) Support WCDMA, LTE TDD, LTE FDD Band. 3) Support SIM slot 4) 802.11n WiFi standard 5) 1* WAN port, 4* LAN ports 6) Support up to 30 users 7) 300Mbps WiFi Spefication Router hardware.

|  | Chipset | MT7628NN |  |  |
| --- | --- | --- | --- | --- |
|  | Power input port | DC 12V---1A |  |  |
|  | WAN Port | 1 x RJ45(10BaseT/100BaseTX with Auto-MDIX IEEE 802.3/802.3u) |  |  |
|  | LAN Port | 4 x RJ45(10BaseT/100BaseTX with Auto-MDIX IEEE 802.3/802.3u) |  |  |
|  | WPS/Reset Buttons | Buttons: WPS ,Long press: reset(Used to restore the factory configuration of the router.) |  |  |
|  | Power button | Router's power switch (not required) |  |  |
|  | LED indication | 1,power supply 2,WIFI Indication 3,3/4G:green light indicates 3G, 4G status 4,WAN Interfaces; 5,LAN1; 6,LAN2; 7,LAN3 8.LAN4 |  |  |
|  | Storage | SDRAM: 64MB Nor Flash: 8MB |  |  |
|  | Wireless parameters |  |  |  |
|  | WIFI Frequency range |  | FCC: 2412-2462MHz(Ch1-Ch11) ETSI: 2412-2472MHz(Ch1-Ch13) |  |
|  | Mobile network frequency |  | FDD: B1 B3 B5 B7 B8 B20 B28 TDD: B38 B39 B40 B41 WCDMA: B1 B8 |  |
|  | Transmission rate |  | 11n:270/243/216/162/108/81/54/27Mbps 135/121.5/108/81/54/40.5/27/13.5Mbps 130/117/104/78/52/39/26/13Mbps 65/58.5/52/39/26/19.5/13/6.5Mbps |  |
|  |  |  | 11g:54/48/36/24/18/12/9/6M(Adaptive) |  |
|  |  |  | 11b:11/5.5/2/1M(Adaptive) |  |
|  | Number of working channels |  | 13 |  |
|  | Spread Spectrum Technology |  | DSSS(Direct Sequence Spread Spectrum) |  |
|  | Data modulation method |  | BPSK, QPSK, CCK and OFDM (BPSK/QPSK/16-QAM/ 64-QAM) |  |
|  | Sensitivity @PER (error packet rate) |  | 270M:-61dBm@10% PER;130M: -61dBm@10% PER;108M:-61dBm@10% PER;54M:-65dBm@10% PER;11M:-85dBm@8% PER;6M:-88dBm@10% PER;1M:-90dBm@8% PER |  |
|  | Transmission distance |  | Up to 50 meters in the room and up to 200 meters in the outdoors (depending on the environment) |  |
|  | RF Power |  | 11n : 15±dBm; 11g : 17±1dBm; 11b : 19±1dBm(Typical value) |  |
|  | LTE related hardware |  |  |  |
|  | SIM Card Slot |  | Rear flip SIM card Slot |  |
|  | Antennas |  | External 4 antenna, corresponding to 4G main antenna, wifi dual antenna |  |
|  | Router software features |  |  |  |
|  | IP Address |  | Static or dynamic |  |

|  | PPP over Ethernet | YES |  |
| --- | --- | --- | --- |
|  | The maximum number of users | 32 (Simultaneous Internet access) |  |
|  | Routing Protocol | Static route, RIP1/RIP2 |  |
|  | Configuration & Management | Based on WEB interface management (HTTP) |  |
|  | DHCP | Support Server and Client |  |
|  | Firewall protection | Built-in NAT firewall |  |